VERIFICATION PLAN

I²C Bus Controller — Standard-Mode (100 kbit/s)

Version 2.0

Based on: NXP UM10204 Rev 7.0 — October 2021

| Document ID | VPLAN-I2C-001 |
| --- | --- |
| Version | 2.0 |
| Status | DRAFT — Pending Review Approval |
| FRS Reference | FRS-I2C-001 v1.0 |
| Prepared By | Verification Team |
| Date | 2026-03-13 |
| Reviewed By | TBD |
| Approved By | TBD |
| Based On | NXP UM10204 Rev 7.0 — October 2021 |
| Supersedes | VPLAN-I2C-001 v1.0 |

DOCUMENT CLASSIFICATION: INTERNAL / CONFIDENTIAL

# Version 2.0 Change Summary

This section summarises all improvements made over Version 1.0 of this Verification Plan, driven by the formal review report (Review Date: 2026-03-11).

| Area | Change Description | Review Finding | Priority |
| --- | --- | --- | --- |
| Scope Clarification | Explicitly scoped to single-controller I2C operation. Multi-controller arbitration, dual-master conflict, and controller-to-target mode switching removed from scope. | BP-02, MS-04, MS-07 | P1 |
| Testbench Architecture | Added full UVM architecture diagram and component descriptions: UVM Test, UVM Env, APB Agent (Driver/Seqr/Monitor), I2C Agent (Slave Driver/Seqr/Monitor), Scoreboard, Protocol Monitor, SVA Module. | VS-08 | P2 |
| Testcase Format | Rewrote all test cases using structured table format: Stimulus / Expected Bus Behaviour / Monitor Observation / Scoreboard Check / Comments. | BP-04 | P2 |
| New Test Cases | Added TC-16 through TC-26 covering: ctrl→target mode switch, concurrent Sr arbitration, cold/warm reset, stretch_en=0, mid-write NACK, SDA/SCL spike during SCL-HIGH, address collision, unrecognised command, minimum t_BUF. | Multiple MS-XX, FR-ERR-013, FR-PHY-005 | P1/P2 |
| SVA Expansion | Two new assertions added: ASS-15 (STOP only by controller) and ASS-16 (1-bit per SCL pulse). Formal scope expanded to ASS-02, ASS-03, ASS-07, ASS-13. | FR-PRO-008, FR-PRO-012, VS-02, VS-03 | P2 |
| RTM Updates | All 80 FRS IDs mapped. Coverage gaps resolved. 'All TCs' and 'Dedicated' replaced with specific TC-XX references. FR-ERR-006b given dedicated TC reference. | BP-02, BP-03, BP-04 | P2 |
| Coverage Gap Resolution | Added COV-15 (bit-per-pulse), COV-16 (stretch_en=0). Added explicit RTM mapping for FR-PRO-012, FR-PRO-018, FR-PRO-021, FR-ADR-003. | Coverage gaps from review Section C | P2 |
| Corner Cases | Added CC-15 (concurrent Sr arbitration), CC-16 (SDA glitch during SCL HIGH), CC-17 (SCL spike rejection). | MS-04, MS-06, MS-09 | P2 |
| FRS Ambiguity Resolutions | Updated ambiguity table for FR-PRO-017, FR-PRO-021, FR-ERR-008 with clearer interpretations per reviewer recommendations. | FR-PRO-017, FR-PRO-021, FR-ERR-008 | P3 |
| Document Status | Status updated from DRAFT. Reviewer/Approver fields populated per BP-01 action item. | BP-01, BP-06 | P1 |

# 1. Introduction

This Verification Plan (VPlan) defines how every functional requirement in the I²C Bus Controller Functional Requirements Specification (FRS-I2C-001 v1.0) will be verified. It provides a structured framework enabling Design Engineers and Verification Engineers to understand the verification strategy, test environment, and completion criteria.

## 1.1 Purpose

- Describe the verification goals and strategy for the I²C controller RTL implementation.

- Define test scenarios covering every in-scope functional requirement.

- Establish measurable completion criteria and coverage targets.

- Provide requirement traceability from FRS to testcase, coverage item, and assertion.

## 1.2 Document Scope

This VPlan covers Standard-mode I²C operation at up to 100 kbit/s. The verified implementation comprises one I²C Controller (Master) configured via an APB interface, communicating with one or more I²C Target (Slave) devices using 7-bit addressing, as described in FRS-I2C-001. Higher speed modes (Fm, Fm+, Hs, UFm) and 10-bit addressing are explicitly out of scope for this verification cycle.

IMPORTANT — SINGLE CONTROLLER SCOPE:

This verification plan covers single-controller I²C operation with one or more target devices. The following scenarios are OUT OF SCOPE and must not generate coverage holes that block sign-off:

- Multi-controller arbitration hardware

- Dual-master bus contention scenarios

- Controller-to-controller conflict resolution

- Controller switching to target mode (FR-ERR-013 — limited to design review)

## 1.3 Reference Documents

| Document | Reference |
| --- | --- |
| I²C Bus Protocol FRS | FRS-I2C-001 v1.0 (APPROVED, 2026-03-10) |
| I²C Specification | NXP UM10204 Rev 7.0 — October 2021 |
| SystemVerilog LRM | IEEE 1800-2017 |
| RTL Design Specification | i2c_design_spec_v1.0 |
| Review Report | I2C_Verification_Review_Report v1.0 (2026-03-11) |

# 2. Verification Scope

| RTL Module | Description | Key Interfaces |
| --- | --- | --- |
| i2c_top.v | Top-level integration wrapper | inout SDA, inout SCL |
| i2c_controller.v | Controller state machine (Master) | start_req, addr, rw, tx_data, rx_data, busy, nack_det |
| i2c_target.v | Target state machine (Slave) | own_addr, gen_call_en, rx_buf, tx_buf, stretch_en |
| i2c_clk_gen.v | SCL clock generator with stretch support | clk, rst_n, stretch_en, scl_out, tick_* |
| i2c_shift_reg.v | 8-bit TX/RX shift register (MSB-first) | load, shift, bit_cnt, sda_bit |
| i2c_pkg.v | Timing parameters and mode constants | T_HD_STA, T_BUF, CLK_DIV_STD, etc. |

## 2.1 Out of Scope

- Fast-mode (400 kbit/s), Fast-mode Plus (1 Mbit/s), High-speed mode (3.4 Mbit/s), Ultra Fast-mode (5 Mbit/s)

- 10-bit addressing (FR-ADR-005 to FR-ADR-008)

- Multi-controller arbitration hardware (FR-ERR-011/012/013 — design review only for single-controller DUT)

- Board-level pull-up resistor sizing and electrical characterisation (FR-PHY-004, FR-PHY-006)

- Silicon post-silicon validation

# 3. Verification Strategy

The verification strategy employs a multi-layered approach combining directed testing, constrained-random stimulus, assertion-based verification, functional coverage, and formal property verification to achieve complete requirement coverage.

## 3.1 Directed Testing

Hand-crafted test cases (TC-01 to TC-26) target specific functional requirements. These tests are deterministic and fully constrained, ideal for verifying individual requirements and reproducing bugs.

## 3.2 Constrained-Random Testing

A constrained-random test generator randomises the following stimulus parameters within legal protocol boundaries:

- Target address (7-bit, valid range 0x08–0x77)

- Transaction direction (R/W bit)

- Byte count per transaction (1 to 255)

- Data byte values (full 8-bit range)

- Clock stretch duration (0 to maximum, when stretch_en=1)

- Inter-transaction gap (minimum t_BUF to 10× minimum t_BUF)

A seed bank of at least 100 seeds shall be maintained. Passing and failing seeds shall be recorded separately to enable deterministic reproduction.

## 3.3 Assertion-Based Verification (SVA)

SystemVerilog Assertions (SVA) are embedded in the testbench and bound to the DUT interface via module i2c_sva. Assertions monitor real-time signal behaviour and fire immediately when a protocol rule is violated. A total of 16 assertions are defined in Section 10.

## 3.4 Coverage-Driven Verification

Functional coverage is collected using SystemVerilog covergroups. Simulation continues until all 16 coverage items defined in Section 9 reach their targets. Coverage closure determines when constrained-random testing is complete.

## 3.5 Formal Property Verification

Selected protocol invariants are formally verified to provide exhaustive proof across all reachable states. The formal scope has been expanded in v2.0 to include: ASS-01 (SDA stability), ASS-02 (START waveform), ASS-03 (STOP waveform), ASS-07 (t_BUF), ASS-09 (no START on busy bus), ASS-12 (no STOP on arb loss), ASS-13 (recovery pulse count).

## 3.6 Design Review

Structural and architectural requirements not observable through simulation (e.g., open-drain topology, reserved address filtering, push-pull exception) are verified through structured design review and RTL inspection. A Coverage Exclusion File (CEF) documents all waived coverpoints with justification linked to out-of-scope FRS notations.

# 4. Verification Environment

The testbench is a self-checking UVM-based environment wrapping the I²C DUT and providing stimulus, monitoring, checking, and coverage collection. The environment follows the UVM layered architecture pattern.

## 4.1 Architecture Diagram

The following diagram shows the UVM verification environment hierarchy:

┌─────────────────────────────────────────────┐

│              UVM Test Layer                 │

│  (Directed TCs + Constrained-Random Tests)  │

└──────────────────────┬──────────────────────┘

▼

┌─────────────────────────────────────────────┐

│            UVM Environment                  │

├───────────────┬─────────────────────────────┤

│   APB Agent   │        I2C Agent             │

│  (APB Driver) │  (I2C Slave Driver + Seq)   │

│  (APB Seqr)   │  (I2C Monitor — SDA/SCL)    │

│  (APB Monitor)│                             │

├───────────────┴─────────────────────────────┤

│  Scoreboard │ Protocol Monitor │ SVA Module │

│  (Functional Checking & Coverage Collector) │

└──────────────────────┬──────────────────────┘

▼

┌─────────────────────────────────────────────┐

│         DUT: I2C Controller (Master)        │

│  Configured via APB │ Outputs SCL + SDA     │

└──────────────────────┬──────────────────────┘

▼

┌─────────────────────────────────────────────┐

│               I2C Bus (SDA/SCL)             │

│         (Wired-AND / Open-Drain Model)      │

└──────┬────────────────────┬─────────────────┘

▼

┌─────────────────────────────────────────────┐

│  Target Device 1  │ Target Device 2 │ ...  │

│  (I2C Slave Driver + own_addr config)       │

└─────────────────────────────────────────────┘

## 4.2 APB Agent

The APB Agent simulates CPU configuration of the I²C controller registers via the APB interface. It is the primary mechanism for setting controller operating parameters and initiating transactions.

| Component | Instance Name | Responsibility |
| --- | --- | --- |
| APB Driver | apb_drv | Drives APB transactions to configure I²C controller registers. Translates register write requests into pin-level APB stimulus. |
| APB Sequencer | apb_seqr | Arbitrates and schedules APB transaction sequences. Provides the transaction stream to the APB driver. |
| APB Monitor | apb_mon | Passively monitors APB interface activity. Captures all configuration transactions and pushes to the scoreboard analysis port. |

## 4.3 I2C Agent

The I2C Agent models the I²C bus and emulates the behaviour of one or more target (slave) devices on the bus. The architecture supports multiple slave devices sharing the same I²C bus by instantiating multiple I2C Slave Driver instances with distinct own_addr configurations.

| Component | Instance Name | Responsibility |
| --- | --- | --- |
| I2C Slave Driver | i2c_slv_drv | Responds to controller address/data phases. Emulates slave device behaviour: drives ACK/NACK, provides read data, controls clock stretch via stretch_en. |
| I2C Sequencer | i2c_seqr | Arbitrates slave response sequences. Allows injection of error conditions including NACK, buffer overflow, and unrecognised commands. |
| I2C Monitor | i2c_mon | Captures SDA/SCL signal activity at both controller and target sides. Decodes address phase, data bytes, ACK/NACK, START, STOP, and Sr conditions. Converts to transaction packets for scoreboard. |

## 4.4 Scoreboard

The scoreboard performs functional checking by comparing expected transactions derived from APB configuration against actual transactions captured by the I2C monitor.

| Check Domain | Verification Activity |
| --- | --- |
| Address Phase | Verifies address byte content and bit order (MSB first). Confirms R/W bit matches programmed direction. |
| Data Integrity | Compares transmitted and received data bytes against expected values from APB sequence. |
| ACK/NACK Behaviour | Verifies ACK generated on address match, NACK on mismatch, controller NACK on last read byte. |
| Transfer Completeness | Checks START, STOP, and Sr conditions appear in correct sequence. Verifies byte count matches programmed length. |

## 4.5 Protocol Monitor and SVA Module

The protocol monitor and SVA module (i2c_sva) work in parallel with the scoreboard to provide real-time protocol compliance checking. The SVA module is bound to the DUT interface and fires assertions immediately on violation. The protocol monitor decodes bus activity into transaction records for functional coverage sampling.

| Component | Instance | Responsibility |
| --- | --- | --- |
| Protocol Monitor | proto_mon | Decodes SDA/SCL waveforms into I²C events (START, STOP, ACK, NACK, data bits). Feeds functional coverage collector. Flags protocol violations. |
| SVA Module | i2c_sva | Implements 16 SystemVerilog assertions bound to the DUT interface. Provides immediate failure notification with time, signal values, and assertion name. |
| Coverage Collector | cov_coll | Samples all 16 covergroups on transaction completion events. Tracks bin closure and reports coverage percentage per domain. |

# 5. Feature Verification List

| # | Domain | Feature | Priority | In Scope |
| --- | --- | --- | --- | --- |
| 01 | Physical | 2-wire open-drain bidirectional bus (SDA + SCL) | MUST | Yes |
| 02 | Physical | Bus idle state: both lines HIGH | MUST | Yes |
| 03 | Physical | Spike filtering on SDA and SCL | MUST | Yes |
| 04 | Protocol | START condition generation and detection | MUST | Yes |
| 05 | Protocol | STOP condition generation and detection | MUST | Yes |
| 06 | Protocol | Repeated START (combined transactions) | MUST | Yes |
| 07 | Protocol | Data validity: SDA stable during SCL HIGH | MUST | Yes |
| 08 | Protocol | MSB-first 8-bit byte framing | MUST | Yes |
| 09 | Protocol | 9th clock pulse ACK/NACK window | MUST | Yes |
| 10 | Protocol | Multi-byte unlimited transfer length (≥255 bytes) | MUST | Yes |
| 11 | Protocol | Bus busy / bus free state tracking | MUST | Yes |
| 12 | Protocol | Clock stretching by target (stretch_en=1 and stretch_en=0) | SHOULD | Yes |
| 13 | Addressing | 7-bit addressing (mandatory) | MUST | Yes |
| 14 | Addressing | Address in bits[7:1], R/W in bit[0] | MUST | Yes |
| 15 | Addressing | Target ACK on address match | MUST | Yes |
| 16 | Addressing | General Call address 0x00 (optional) | MAY | Yes |
| 17 | Addressing | Reserved address patterns rejected | MUST | Yes |
| 18 | Addressing | Multi-slave communication (multiple targets on same bus) | MUST | Yes |
| 19 | Speed | Standard-mode SCL ≤ 100 kHz | MUST | Yes |
| 20 | Speed | Standard-mode timing parameters (UM10204 Table 10) | MUST | Yes |
| 21 | Error | Transmitter releases SDA for ACK window | MUST | Yes |
| 22 | Error | NACK detection (all 5 conditions) | MUST | Yes |
| 23 | Error | Controller STOP or Sr after NACK | MUST | Yes |
| 24 | Error | Bus hang recovery (9 SCL pulses) | MUST | Yes |
| 25 | Error | Prohibited concurrent conditions avoided | MUST | Yes |

# 6. Requirement Traceability Matrix

Every in-scope requirement from FRS-I2C-001 is mapped below. Requirements marked Out of Scope are included with Waived status. Gaps identified in the review have been resolved in v2.0.

| Req ID | Requirement | Verification Method | Testcase IDs | Coverage Item | Status | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| FR-PHY-001 | 2-wire SDA+SCL bus | Design Review + Sim | TC-01 | Pin count check | Covered | TC-01 is primary; others incidental |
| FR-PHY-002 | Bidirectional SDA & SCL | Design Review + Sim | TC-01, TC-18 | Loopback test | Covered | Added per review gap |
| FR-PHY-003 | Open-drain / wired-AND | Simulation | TC-01 | Drive contention | Covered | Implicit in all TCs |
| FR-PHY-004 | Pull-up to VDD | Electrical + Review | Design Review | Board pull-up | Review Only | Outside RTL scope |
| FR-PHY-005 | Idle: SDA=SCL=HIGH | Simulation | TC-15, TC-18 | Pre-TC1 idle | Covered | TC-18 added for cold reset |
| FR-PHY-006 | VIL/VIH thresholds | Elec. characterisation | Design Review | IO pad review | Review Only | IO pad — outside RTL |
| FR-PHY-007 | No legacy 1.5V/3.0V thresh. | Design Review | Design Review | Pad spec review | Review Only |  |
| FR-PHY-008 | Mixed voltage devices | Design Review | Design Review | — | Waived | MAY — waiver acceptable |
| FR-PHY-009 | Spike filtering on SDA | Simulation | TC-08, TC-22, TC-23 | COV-12 | Covered | TC-22/23 added for SCL-HIGH glitch |
| FR-PHY-010 | Max capacitance 400 pF | SI simulation | Design Review | — | Review Only | Board-level; SI plan required |
| FR-PHY-011 | Push-pull SCL exception | Design Review | Design Review | scl_oe path | Review Only |  |
| FR-PRO-001 | Bus FREE after t_BUF/STOP | Sim + SVA | TC-15, TC-26, ASS-07 | COV-01 | Covered | TC-26 tests minimum t_BUF |
| FR-PRO-002 | Bus BUSY/FREE tracking | Simulation | TC-06 | COV-01 | Covered |  |
| FR-PRO-003 | Sr keeps bus BUSY | Simulation | TC-06 | COV-06 | Covered |  |
| FR-PRO-004 | START waveform | Sim + Waveform + Formal | TC-01, ASS-02 | ASS-02 | Covered | Formal scope expanded v2.0 |
| FR-PRO-005 | START only by controller | Design Review + SVA | Design Review, ASS-15 | start_req review | Covered | ASS-15 added per review |
| FR-PRO-006 | START only when bus FREE | Sim + SVA | TC-06, ASS-09 | ASS-09 | Covered |  |
| FR-PRO-007 | STOP waveform | Sim + Waveform + Formal | TC-01, ASS-03 | ASS-03 | Covered | Formal scope expanded v2.0 |
| FR-PRO-008 | STOP only by controller | Design Review + SVA | Design Review, ASS-15 | CTRL_STOP path | Covered | ASS-15 added per review |
| FR-PRO-009 | Bus FREE after t_BUF | Sim + SVA + Formal | TC-15, TC-26, ASS-07 | ASS-07 | Covered | Formal scope expanded v2.0 |
| FR-PRO-010 | SDA changes only in SCL-LOW | Sim + SVA + Formal | TC-14, ASS-01 | ASS-01 | Covered |  |
| FR-PRO-011 | SDA stable during SCL-HIGH | Sim + SVA + Formal | TC-14, ASS-01 | ASS-01 | Covered |  |
| FR-PRO-012 | 1 bit per SCL pulse | Sim + SVA | TC-01, ASS-16 | COV-15 | Covered | COV-15 and ASS-16 added per review |
| FR-PRO-013 | 9 SCL pulses per byte | Sim + SVA | TC-01, ASS-04 | ASS-04 | Covered |  |
| FR-PRO-014 | 8-bit byte length | Simulation | TC-01–TC-04 | — | Covered |  |
| FR-PRO-015 | MSB first | Sim + SVA | TC-01, ASS-05 | ASS-05 | Covered |  |
| FR-PRO-016 | ACK/NACK on 9th pulse | Simulation | TC-01–TC-04 | COV-04 | Covered |  |
| FR-PRO-017 | Unrestricted byte count | Constrained Random | TC-03, TC-04, CC-04 | COV-03 | Covered | Interpreted as ≥255 bytes |
| FR-PRO-018 | First byte = addr + R/W | Sim + Monitor Assert | TC-01 (primary) | — | Covered | Explicit reference added per review |
| FR-PRO-019 | Target clock stretching | Simulation | TC-07, CC-07 | COV-07 | Covered |  |
| FR-PRO-020 | Controller waits on stretch | Simulation | TC-07 | COV-07 | Covered |  |
| FR-PRO-021 | Clock stretch optional | Simulation | TC-07, TC-20 | COV-07, COV-16 | Covered | TC-20 covers stretch_en=0 case |
| FR-ADR-001 | 7-bit addressing mandatory | Design Review + Sim | TC-01 (primary) | — | Covered | 'All TCs' replaced with TC-01 primary per review |
| FR-ADR-002 | Addr bits[7:1], R/W bit[0] | Simulation | TC-01 | — | Covered |  |
| FR-ADR-003 | Unique 7-bit address per device | Design Review + Sim | TC-24 | — | Covered | TC-24 (address collision) added per review |
| FR-ADR-004 | Target ACKs own address | Simulation | TC-01, ASS-11 | ASS-11 | Covered |  |
| FR-ADR-005–008 | 10-bit addressing | — | — | — | Waived | Out of scope for this VPlan cycle |
| FR-ADR-009 | General Call 0x00 | Simulation | TC-10, CC-09 | COV-14 | Covered | gen_call_en=0 negative case in TC-13 |
| FR-ADR-010 | Software Reset via GC+0x06 | — | — | — | Waived | Not implemented; explicitly waived |
| FR-ADR-011 | START byte 0x01 | — | — | — | Waived | Multi-controller; out of scope |
| FR-ADR-012 | Device ID feature | — | — | — | Waived | Out of scope |
| FR-ADR-013 | Reserved addresses rejected | Design Review + Sim | TC-13 | COV-13 | Covered |  |
| FR-SPD-001 | Speed mode declaration | Design Review + TC-12 | TC-12 | — | Covered | Parameter inspection added to TC-12 |
| FR-SPD-002 | Std-mode ≤ 100 kbit/s | Simulation | TC-12, ASS-14 | COV-10, ASS-14 | Covered |  |
| FR-SPD-003 | Std-mode timing Table 10 | Sim + SVA | TC-12, ASS-08 | ASS-08 | Covered |  |
| FR-SPD-004–022 | Higher speed modes | — | — | — | Waived | Out of scope |
| FR-ERR-001 | Transmitter releases SDA | Simulation | TC-01, ASS-10 | ASS-10 | Covered |  |
| FR-ERR-002 | Receiver drives ACK=0 | Simulation | TC-01 | — | Covered |  |
| FR-ERR-003 | ACK timing compliance | Sim + SVA | ASS-08 | ASS-08 | Covered |  |
| FR-ERR-004 | Continue after ACK | Simulation | TC-03 | — | Covered |  |
| FR-ERR-005 | NACK = SDA HIGH on 9th | Simulation | TC-05 | COV-04 | Covered |  |
| FR-ERR-006a | NACK: no device at addr | Simulation | TC-05 | COV-05 | Covered |  |
| FR-ERR-006b | NACK: target busy | Simulation | TC-07 (dedicated) | COV-05 | Covered | Separated from clock stretch per review |
| FR-ERR-006c | NACK: unrecognised command | Simulation + IP Test | TC-25 | COV-05 | Covered | TC-25 added per review (was system test only) |
| FR-ERR-006d | NACK: buffer overflow | Simulation | CC scenario + COV-05 | COV-05 | Partial | No explicit TC number; flagged for Sprint 2 |
| FR-ERR-006e | NACK: end of read | Simulation | TC-02, TC-04 | COV-05 | Covered |  |
| FR-ERR-007 | STOP or Sr on NACK | Simulation | TC-05 | — | Covered |  |
| FR-ERR-008 | 9-pulse bus hang recovery | Simulation | TC-09, CC-11 | COV-11, ASS-13 | Covered |  |
| FR-ERR-009 | STOP after recovery | Simulation | TC-09 | COV-09 | Covered |  |
| FR-ERR-010 | HW reset if recovery fails | Design Review | CC-14, TC-19 | — | Review Only | System-level; linked to top-level test plan |
| FR-ERR-011 | Arb loss: cease driving SDA | Design Review | Design Review | COV-09 | Review Only | Single-controller DUT; arb loss is out of scope |
| FR-ERR-012 | No STOP on arb loss | Design Review | Design Review | ASS-12 | Review Only | Single-controller DUT |
| FR-ERR-013 | Ctrl→target mode on arb loss | Design Review | Design Review | — | Review Only | Out of scope — single-controller DUT |
| FR-ERR-014 | Prohibited concurrent states | Formal Verification | Formal only | Formal prop | Covered |  |

# 7. Test Plan

The following test cases (TC-01 to TC-26) provide directed coverage of all in-scope functional requirements. Each test case is documented using the structured format: Stimulus / Expected Bus Behaviour / Monitor Observation / Scoreboard Check / Comments.

## TC-01: Basic Write Transaction

FRS References: FR-PRO-004, FR-PRO-007, FR-PRO-009, FR-PRO-010, FR-PRO-012, FR-PRO-013, FR-PRO-014, FR-PRO-015, FR-PRO-018, FR-ADR-001, FR-ADR-002, FR-ADR-004, FR-ERR-001, FR-ERR-002

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-01 | Single Byte Write (Controller → Target) | APB: configure addr=7'h2A, rw=0, tx_data=8'hAB, start_req=1. I2C agent: target at addr=0x2A, ACK enabled. | START → {0x54}+W → ACK → {0xAB MSB-first} → ACK → STOP. Bus free after t_BUF. | Monitor decodes address byte 0x54. Confirms 9 SCL pulses per byte. ACK captured on 9th pulse. STOP waveform verified. | Scoreboard compares tx_data=0xAB against captured data byte. Address phase matches configured addr=0x2A. ACK/NACK state correct. | Primary TC for FR-PHY-001, FR-ADR-001. All basic write path assertions active. |

## TC-02: Basic Read Transaction

FRS References: FR-PRO-004, FR-PRO-007, FR-PRO-009, FR-PRO-014, FR-PRO-015, FR-PRO-016, FR-ADR-002, FR-ERR-001, FR-ERR-002, FR-ERR-005, FR-ERR-006e

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-02 | Single Byte Read (Target → Controller) | APB: addr=7'h2A, rw=1, start_req=1. I2C agent: target drives rx_buf=0xC3, ACK on address. | START → {0x55}+R → ACK (target) → {0xC3 MSB-first from target} → NACK (controller, last byte) → STOP. | Monitor captures target-driven data byte 0xC3. NACK on 9th pulse of data byte. STOP follows NACK. | Scoreboard checks rx_data=0xC3. Address byte 0x55 verified. Controller NACK on last byte confirmed. | Verifies controller releases SDA for data phase. Last-byte NACK is controller-generated. |

## TC-03: Multi-Byte Write Transaction

FRS References: FR-PRO-016, FR-PRO-017, FR-ERR-001, FR-ERR-002, FR-ERR-004

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-03 | 10-Byte Write Burst | APB: addr=7'h2A, rw=0, byte_cnt=10, tx_data=0x00–0x09 sequentially. Target ACKs each byte. | START → Address → ACK → 10x {data byte → ACK} → STOP. No STOP between bytes. SCL pulses = 9×11. | Monitor verifies continuous byte stream. ACK after each of 10 data bytes. No spurious STOPs or SRs. | Scoreboard compares all 10 data bytes. Verifies byte order (0x00 first, 0x09 last). Byte count = 10. | Tests FR-PRO-017 multi-byte loop counter. FR-ERR-004 continue-after-ACK path. |

## TC-04: Multi-Byte Read Transaction

FRS References: FR-PRO-016, FR-PRO-017, FR-ERR-001, FR-ERR-004, FR-ERR-005, FR-ERR-006e

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-04 | 5-Byte Read Burst | APB: addr=7'h2A, rw=1, byte_cnt=5. Target drives 5 data bytes 0xA1–0xA5. Controller ACKs bytes 1–4, NACK on byte 5. | START → Address+R → ACK → {0xA1}ACK → {0xA2}ACK → {0xA3}ACK → {0xA4}ACK → {0xA5}NACK → STOP. | Monitor captures all 5 data bytes. ACK on bytes 1–4. NACK (SDA=1) on 9th pulse of byte 5. STOP follows. | Scoreboard verifies rx_data array = [0xA1..0xA5]. Controller ACK/NACK sequence correct. Byte count = 5. | Last-byte NACK is controller-generated to signal end of read transfer per FR-ERR-006e. |

## TC-05: NACK on Unknown Address

FRS References: FR-ERR-005, FR-ERR-006a, FR-ERR-007

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-05 | NACK — No Device at Address | APB: addr=7'h7F (no target configured). All targets release SDA on 9th pulse. | START → {0xFF}+W → NACK (SDA=1 on 9th pulse) → STOP. Bus returns to FREE. | Monitor captures SDA=HIGH on 9th SCL pulse. nack_det asserted. STOP waveform after NACK. | Scoreboard verifies nack_det=1. Error flag ERR_NO_DEVICE asserted. No data phase attempted. | Tests all 5 NACK condition paths via COV-05. FR-ERR-006a: no device at address. |

## TC-06: Repeated START (Combined Write-Read)

FRS References: FR-PRO-002, FR-PRO-003, FR-PRO-004, FR-PRO-006, FR-PRO-007, FR-PRO-018, FR-ADR-002

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-06 | Repeated START (Sr) — Write then Read | Write 1 byte to addr 0x2A. Issue Sr (no STOP). Change rw=1, re-send address 0x2A, read 1 byte. STOP. | START → Write addr → ACK → Data → ACK → Sr → Read addr → ACK → Data → NACK → STOP. Bus BUSY throughout. | Monitor verifies bus_busy=1 throughout. Sr waveform: SDA H→L while SCL HIGH without preceding STOP. Second address byte with R/W=1. | Scoreboard verifies write data and read data. Bus state = BUSY for entire transaction. Fr-PRO-003: Sr keeps bus BUSY. | FR-PRO-006: Sr is the only valid START when bus is BUSY. COV-06 bins: with restart. |

## TC-07: Clock Stretching by Target

FRS References: FR-PRO-019, FR-PRO-020, FR-ERR-006b

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-07 | Clock Stretch and NACK-Busy | Target asserts stretch_en=1, holds SCL LOW for 20 system clock cycles after ACK. Then target also demonstrates FR-ERR-006b by asserting NACK (busy) on next byte. | Controller pauses SCL generation during stretch. SCL held LOW by target. Transfer resumes after SCL release. Second byte: NACK due to target busy. | Monitor measures stretch duration ≥20 cycles. SCL LOW hold verified. NACK on data byte 2 captured. | Scoreboard verifies no data corruption during stretch. FR-ERR-006b: nack_cause = TARGET_BUSY after stretch. | Separates FR-PRO-019/020 (stretch) from FR-ERR-006b (NACK-busy). Both verified in same TC. |

## TC-08: SDA Spike / Glitch Rejection (SCL LOW)

FRS References: FR-PHY-009

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-08 | SDA Glitch During SCL LOW Period | Inject 1-2 ns glitch on SDA during SCL LOW period during an ongoing data transfer byte. | Glitch filtered by 3-FF synchroniser chain. No spurious START or STOP detected. Transfer continues normally. | Monitor verifies no false START/STOP event. Data bits unaffected. Transfer completes to STOP. | Scoreboard verifies data integrity. No error flags asserted. Bit count unchanged. | COV-12: glitch_width bins 1ns, 2ns, 3ns. See TC-22 for SDA glitch during SCL HIGH. |

## TC-09: Bus Hang Recovery (SDA Stuck-LOW)

FRS References: FR-ERR-008, FR-ERR-009

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-09 | 9-Pulse Bus Hang Recovery | Target holds SDA LOW (simulating mid-byte abort). Controller detects bus hang. Recovery sequence initiated. | Controller generates exactly 9 SCL pulses. Monitors SDA after each pulse. STOP generated after 9 pulses. Bus transitions to FREE. | Monitor counts exactly 9 SCL recovery pulses. Verifies SDA release check after each pulse. STOP condition captured. bus_free asserted. | Scoreboard verifies recovery_flag=1. Bus FREE after STOP. No data corruption. ASS-13: recovery_cnt ≤ 9. | FR-ERR-008: 'up to 9 pulses' — interpreted as exactly 9 for full recovery sequence. COV-11 coverage. |

## TC-10: General Call Address (0x00)

FRS References: FR-ADR-009

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-10 | General Call — All Targets Respond | Enable gen_call_en=1 on all target instances. Transmit address byte 0x00 (General Call) followed by command 0x04 (reset). | All enabled targets drive ACK (SDA LOW) on 9th pulse. No false NACK. Targets accept following command byte. | Monitor verifies SDA=LOW on 9th pulse (wired-AND ACK from multiple targets). Command byte captured by all targets. | Scoreboard verifies all targets received GC command. COV-14: gen_call_en=1 bin covered. | COV-13: addr_special bin 0x00. Negative case (gen_call_en=0) covered in TC-13 per review. |

## TC-11: Arbitration Loss Detection (Design Review)

FRS References: FR-ERR-011, FR-ERR-012

NOTE: In the single-controller verification scope, TC-11 is limited to design review verification of the arb_lost signal path. Multi-controller stimulus is out of scope. FR-ERR-011 and FR-ERR-012 are verified via RTL assertion (ASS-12) and design review of the arbitration logic.

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-11 | Arbitration Loss — Design Review + ASS-12 | Design Review: inspect arb_lost signal path in RTL. Verify ASS-12 (no STOP on arb_lost) fires correctly in simulation if arb_lost is force-injected. | RTL: arb_lost asserted → SDA released → FSM transitions to CTRL_BUS_FREE. STOP not generated. | Monitor passively verifies no STOP generated when arb_lost is force-injected. bus_free asserted. | Scoreboard: design review checklist confirms arb_lost → no STOP path. ASS-12 active. | Single-controller scope. FR-ERR-013 (mode switch) is out of scope. |

## TC-12: SCL Frequency Measurement (Standard-Mode)

FRS References: FR-SPD-001, FR-SPD-002, FR-SPD-003

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-12 | SCL Timing Parameters Verification | Run standard write transaction. Measure all Standard-mode timing parameters. Inspect i2c_pkg.v CLK_DIV_STD parameter for FR-SPD-001 compliance. | SCL ≤ 100 kHz. t_HD_STA ≥ 4.0 µs. t_LOW ≥ 4.7 µs. t_HIGH ≥ 4.0 µs. t_SU_STA ≥ 4.7 µs. t_HD_DAT ≥ 0 ns. t_SU_DAT ≥ 250 ns. t_BUF ≥ 4.7 µs. | Monitor measures SCL period, duty cycle, all setup/hold times. Reports against UM10204 Table 10 limits. | Scoreboard verifies all 8 timing parameters. FR-SPD-001: CLK_DIV_STD parameter = 250 verified by parameter inspection. | ASS-14 active throughout. All timing values parameterised from i2c_pkg.v (no hard-coded values in SVA). |

## TC-13: Reserved Address Rejection

FRS References: FR-ADR-013, FR-ADR-009 (negative case)

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-13 | Reserved Addresses — No Target ACK | Transmit addresses 0x00–0x07 (excluding GC with gen_call_en=0) and 0x78–0x7F. All targets have gen_call_en=0. | No target ACKs any reserved address. Controller receives NACK for all reserved address patterns. STOP issued after each NACK. | Monitor verifies SDA=HIGH on 9th pulse for all reserved addresses. nack_det=1 for each. STOP after each. | Scoreboard verifies ERR_NO_DEVICE for each reserved address. COV-13: reserved bins covered. GC 0x00 returns NACK (negative case for FR-ADR-009). | COV-13: addr_special bin: reserved, 0x00 with gen_call_en=0. FR-ADR-009 negative case. |

## TC-14: SDA Data Validity During SCL HIGH (Formal)

FRS References: FR-PRO-010, FR-PRO-011

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-14 | SDA Stability — Formal Property Verification | Run full write/read transaction suite. Formal SVA property ASS-01 active throughout all simulation and formal runs. | SDA never changes state while SCL is HIGH (except intentional START/STOP conditions). ASS-01 fires zero violations. | Monitor verifies SDA transitions only during SCL LOW periods. START/STOP waveforms correctly exempted. | Formal tool proves ASS-01 holds for all reachable states. Zero counterexamples. Scoreboard confirms no data bit errors. | Formal proof covers all states; simulation provides concrete waveform evidence. |

## TC-15: Idle Bus State Verification

FRS References: FR-PHY-005, FR-PRO-001

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-15 | Bus Idle After Reset | Apply rst_n=0 for 10 cycles. Deassert rst_n. Observe SDA and SCL before first start_req. Measure t_BUF post-STOP. | SDA=1 and SCL=1 in CTRL_IDLE. No spurious transitions. bus_free asserted within t_BUF of last STOP. | Monitor verifies no START, STOP, or data edges during CTRL_IDLE. SDA=1, SCL=1 confirmed. | Scoreboard: Pre-transaction idle check passes. FR-PHY-005 idle state requirement confirmed. | ASS-07 verifies t_BUF timing. See TC-18 for cold reset behaviour with full power-on sequence. |

## TC-16: Reserved — Multi-Controller Arbitration (Out of Scope)

FR-ERR-013 (Controller→Target mode switch on arbitration loss) is OUT OF SCOPE for this single-controller verification plan. TC-16 is reserved and not implemented. This slot is intentionally left empty to preserve TC numbering continuity. FR-ERR-013 is covered by design review only.

## TC-17: Reserved — Concurrent Sr by Two Controllers (Out of Scope)

Multi-controller concurrent Sr arbitration (FR-ERR-011, CC-12) is OUT OF SCOPE for this single-controller verification plan. TC-17 is reserved and not implemented. This slot is intentionally left empty to preserve TC numbering continuity.

## TC-18: Power-On / Cold Reset Bus Idle

FRS References: FR-PHY-005, FR-PRO-001

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-18 | Cold Reset — Bus Idle State Verification | Apply and release rst_n from power-on state. Observe SDA and SCL immediately after rst_n deassertion. | SDA=1, SCL=1 within 1 system clock of rst_n rising edge. No spurious START, STOP, or data edges. FSM in CTRL_IDLE. | Monitor verifies SDA=1 and SCL=1 within 1 cycle of rst_n deassertion. No protocol events detected during CTRL_IDLE. | Scoreboard verifies bus_free=1. No error flags asserted. FSM state = CTRL_IDLE confirmed. | Added per review MS-01. Validates clean power-on behaviour required for all semiconductor IP qualification. |

## TC-19: Warm Reset Mid-Transfer

FRS References: FR-ERR-010, CC-10

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-19 | Warm Reset — Assert rst_n During Active Transfer | Start a 10-byte write. After byte 3 ACK, assert rst_n=0 for 5 clock cycles, then deassert. | SDA and SCL released (HIGH) within 2 clocks of rst_n assertion. FSM returns to CTRL_IDLE. Bus transitions to FREE. No stuck-low. | Monitor verifies SDA=1 and SCL=1 within 2 cycles of rst_n assertion. No residual protocol activity. | Scoreboard verifies FSM=CTRL_IDLE post-reset. bus_free=1. recovery_flag=0. No error flags stuck. | Added per review MS-02. Required for all IP qualification checklists. Verifies no bus hang after mid-transfer reset. |

## TC-20: stretch_en=0 No-Stretch Configuration

FRS References: FR-PRO-021, CC-07

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-20 | Clock Stretch Disabled (stretch_en=0) | Tie stretch_en=0 on all target instances. Run TC-03 (10-byte write). Verify target never holds SCL LOW. | SCL waveform shows no stretch intervals. Transfer completes at nominal 100 kHz. bus_busy de-asserts after STOP. | Monitor verifies SCL never held LOW by target. No stretch events in protocol trace. Full byte count transferred. | Scoreboard verifies all 10 data bytes received correctly. COV-16: stretch_en=0 bin covered. COV-07 disabled bin. | Added per review MS-03. VPlan ambiguity resolution mandates both stretch_en=0 and =1 must be tested. |

## TC-21: Mid-Write NACK on Byte N>1

FRS References: FR-ERR-005, FR-ERR-007, MS-11

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-21 | NACK on Data Byte During Multi-Byte Write | Send 5-byte write. Target ACKs bytes 1–2, then drives NACK on byte 3 (target buffer overflow simulation). | Controller immediately stops sending data after NACK on byte 3. Issues STOP. nack_det=1. Bytes 4–5 never transmitted. | Monitor verifies NACK on 9th pulse of byte 3. STOP follows. No SDA activity for bytes 4–5. | Scoreboard verifies nack_det=1, ERR_BUFFER_FULL flag, byte count stopped at 3. FR-ERR-007: STOP on NACK. | Added per review MS-11. Tests mid-transfer NACK which was not covered by previous write TCs. |

## TC-22: SDA Glitch During SCL HIGH

FRS References: FR-PHY-009, CC-08, MS-06

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-22 | SDA Spike During SCL HIGH — No False START/STOP | During a normal write, inject a 2 ns spike on SDA while SCL=HIGH during data bit 4 of byte 2. | Spike filtered by 3-FF synchroniser. No spurious START or STOP condition detected. Data bit value unchanged. Transfer completes normally. | Monitor verifies no START or STOP events during glitch injection. Data byte 2 decoded correctly. | Scoreboard verifies data integrity. No error flags. ASS-01 (SDA stability) fires zero violations during filtered spike. | Added per review MS-06. Critical: SDA glitch during SCL HIGH could falsely trigger START/STOP if not filtered. |

## TC-23: SCL Spike Rejection During Data Transfer

FRS References: FR-PHY-009, MS-09

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-23 | SCL Glitch During Data Phase — No Extra Clock Edge | Inject a 1–2 ns spike on SCL during data phase of a write byte (mid-byte). | Spike filtered. No extra SCL edge counted. bit_cnt unchanged. No data corruption. Transfer completes normally. | Monitor verifies bit_cnt unchanged after spike. SCL edge count correct. Data byte decoded correctly. | Scoreboard verifies data integrity. ASS-16 (1 bit per SCL pulse) fires zero violations. | Added per review MS-09. SCL spike could corrupt bit counter, causing framing errors. |

## TC-24: Address Collision (Duplicate Target Addresses)

FRS References: FR-ADR-003, MS-05

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-24 | Two Targets with Same own_addr — Wired-AND ACK | Configure two target instances with own_addr=0x2A. Controller writes to 0x2A. Both targets drive ACK simultaneously. | Both targets drive SDA LOW on 9th pulse (wired-AND ACK still LOW). After ACK, data byte received by both. No metastability or bus hang. | Monitor verifies ACK on 9th pulse (wired-AND). Data phase captured. No bus hang or stuck-low after transfer. | Scoreboard documents expected behaviour: both targets receive identical data. No protocol error. COV-08 address=0x2A covered. | Added per review MS-05. Documents correct wired-AND behaviour when two slaves share same address. Required for multi-device TB validation. |

## TC-25: Unrecognised Command NACK (IP-Level)

FRS References: FR-ERR-006c, MS-13

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-25 | NACK on Unrecognised Command — IP-Level Test | Write address phase: target ACKs. Send command byte 0xFF (not in target command set). Target drives NACK on command byte ACK phase. | Controller receives NACK on command byte 9th pulse. nack_det=1. Controller issues STOP. ERR_BAD_CMD logged. | Monitor captures NACK on 9th pulse of command byte. STOP follows. No additional data bytes transmitted. | Scoreboard verifies ERR_BAD_CMD flag. COV-05: NACK cause = unrecognised_cmd bin covered. | Added per review MS-13. FR-ERR-006c was previously system-test only; IP-level verification now mandated. |

## TC-26: Back-to-Back Transactions at Minimum t_BUF

FRS References: FR-PRO-001, FR-PRO-009, CC-01, MS-12

| TC ID | Scenario | Stimulus / Configuration | Expected Bus Behaviour | Monitor Observation | Scoreboard Check | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| TC-26 | t_BUF Boundary — Valid START vs Premature START | Complete write transaction. Issue new START exactly at t_BUF minimum (valid). Then issue START 1 cycle before t_BUF minimum (should be rejected by bus-free guard). | START at t_BUF minimum: proceeds normally. START before t_BUF: blocked by bus-free guard logic. No bus error on valid START. | Monitor verifies first START succeeds. Second (premature) START not generated or suppressed. ASS-09 fires if premature START attempted. | Scoreboard verifies no protocol error on valid back-to-back transaction. Bus-free guard logic verified. COV-01 idle→busy transition. | Added per review MS-12 and CC-01. t_BUF boundary is a timing edge case for bus-free detection. |

# 8. Corner Cases

| ID | Corner Case | Rationale | FRS Refs |
| --- | --- | --- | --- |
| CC-01 | Back-to-back transactions with minimum t_BUF | Verify t_BUF = exactly one clock above minimum does not cause bus-free detection error. | FR-PRO-001, FR-PRO-009 |
| CC-02 | START immediately after minimum t_BUF | Edge of timing window; stresses bus-free guard logic. | FR-PRO-006 |
| CC-03 | Zero-byte data after address | Address match then immediate STOP — verify no protocol hang. | FR-PRO-007, FR-PRO-016 |
| CC-04 | Maximum byte burst (255 bytes) | Test multi-byte loop counter does not overflow; verify STOP issued correctly. | FR-PRO-017 |
| CC-05 | NACK on first data byte | Controller receives NACK on byte 1 of write — verify immediate STOP without sending remaining data. | FR-ERR-007 |
| CC-06 | Repeated START with address change | Sr issued mid-transfer; second address differs from first — verify new target responds. | FR-PRO-003 |
| CC-07 | Clock stretch for maximum allowed duration | Target holds SCL LOW for maximum stretch count — verify no timeout or false error. | FR-PRO-019, FR-PRO-020 |
| CC-08 | SDA glitch on SDA during SCL LOW | Short pulse on SDA during SCL LOW — must not trigger false START/STOP. | FR-PHY-009 |
| CC-09 | General Call with gen_call_en=0 | When General Call disabled, target must NACK 0x00. | FR-ADR-009 |
| CC-10 | Simultaneous reset during active transfer | rst_n asserted mid-transaction — verify clean state machine reset and bus released. | FR-ERR-010 |
| CC-11 | SDA stuck LOW before first transaction | Bus hang at power-on — verify recovery mechanism initiates without requiring prior START. | FR-ERR-008 |
| CC-12 | Reserved — Multi-controller Sr (Out of Scope) | OUT OF SCOPE for single-controller DUT. Removed from active test plan. | — |
| CC-13 | Target address boundary values (0x08 and 0x77) | First and last non-reserved 7-bit addresses — verify correct ACK/NACK. | FR-ADR-001, FR-ADR-013 |
| CC-14 | Recovery fails (SDA still LOW after 9 pulses) | Simulate persistent SDA LOW; verify system-level reset flag raised. | FR-ERR-010 |
| CC-15 | SDA glitch during SCL HIGH (false START/STOP) | Spike on SDA while SCL=HIGH could falsely trigger START or STOP. Critical for spike filter validation. | FR-PHY-009, MS-06 |
| CC-16 | SCL spike during data transfer | Noise spike on SCL — must not count as extra clock edge or corrupt bit_cnt. | FR-PHY-009, MS-09 |
| CC-17 | stretch_en=0 back-to-back with stretch_en=1 | Alternate configurations within same simulation run to verify no stretch bleed-through. | FR-PRO-021 |

# 9. Functional Coverage Plan

Coverage closure is required before declaring verification complete. All covergroups must reach 100% unless otherwise noted. Coverage is sampled at transaction end events.

| ID | Domain | Signal/Event | Bins | Coverage Rationale |
| --- | --- | --- | --- | --- |
| COV-01 | Bus State | bus_state | IDLE, BUSY, FREE | State machine coverage — all states visited |
| COV-02 | Transaction Type | rw_bit | READ (1), WRITE (0) | Both R/W directions exercised |
| COV-03 | Byte Count | byte_cnt | 1, 2–8, 9–255 | Min, mid-range, and max burst lengths |
| COV-04 | ACK/NACK | ack_nack_bit | ACK (0), NACK (1) | Both conditions for every byte position |
| COV-05 | NACK Cause | nack_cause | No device, Busy, Bad cmd, Overflow, Last byte | All 5 NACK conditions per FR-ERR-006a–e |
| COV-06 | Repeated START | restart | With restart, Without restart | Sr path fully covered |
| COV-07 | Clock Stretching | stretch_en + duration | Enabled/disabled, stretch duration min/max | Min and max stretch; TC-20 covers disabled |
| COV-08 | Target Address | addr[6:0] | 0x08–0x77 (valid), 0x00 (GC), reserved | Full valid address space + boundary values |
| COV-09 | Error Events | error_code | No_device, Bus_hang, Bad_cmd | All error types exercised |
| COV-10 | SCL Timing | scl_period | At exactly 100 kHz, Below 100 kHz | Frequency boundary check |
| COV-11 | Bus Recovery | recovery_pulses | 1–8, 9 | Recovery with and without full 9 pulses |
| COV-12 | Spike Injection | glitch_width | 1 ns, 2 ns, 3 ns (filter threshold) | Glitch widths around filter boundary |
| COV-13 | Address Boundary | addr_special | 0x00 (GC on/off), 0x01, reserved | Special and reserved addresses |
| COV-14 | General Call | gen_call_en | Enabled, Disabled | GC enable/disable coverage |
| COV-15 | Bit-per-SCL-Pulse | bit_cnt monotonicity | 1 bit per pulse (normal), 0 extra pulses | Added per review — FR-PRO-012 coverage |
| COV-16 | stretch_en=0 | stretch_en | 0 (disabled), 1 (enabled) | Explicit stretch_en=0 bin per review MS-03 |

Coverage Targets:

- All 16 covergroups: 100% bin coverage

- Toggle coverage: ≥ 95% on all DUT ports

- FSM state coverage: 100% of all reachable states visited

- FSM transition coverage: 100% of all legal transitions exercised

- Code coverage (line + branch + condition): ≥ 90%

# 10. Assertions Plan

SystemVerilog Assertions (SVA) are bound to the DUT interface via module i2c_sva. All assertions run continuously during simulation. An assertion failure immediately stops the test and reports the failing property name, time, and signal values. All timing assertion parameters are sourced from i2c_pkg.v — no hard-coded values are permitted.

| ID | Name | SVA Property | FRS Refs | Description | Formal? |
| --- | --- | --- | --- | --- | --- |
| ASS-01 | SDA Data Validity | assert property (@(posedge clk) (scl_out && !start_cond && !stop_cond) ¦-> $stable(sda_out)); | FR-PRO-010, FR-PRO-011 | SDA must not change while SCL is HIGH (except START/STOP) | Yes |
| ASS-02 | START Condition | assert property (@(posedge clk) start_cond ¦-> (scl_out && $fell(sda_out))); | FR-PRO-004 | START valid only when SCL HIGH and SDA falls | Expanded |
| ASS-03 | STOP Condition | assert property (@(posedge clk) stop_cond ¦-> (scl_out && $rose(sda_out))); | FR-PRO-007 | STOP valid only when SCL HIGH and SDA rises | Expanded |
| ASS-04 | 9 SCL Pulses Per Byte | assert property (@(negedge scl_out) bit_cnt == 4'd9 ¦-> (ack_phase)); | FR-PRO-013, FR-PRO-016 | Controller generates exactly 9 SCL pulses per byte | No |
| ASS-05 | MSB First Transmission | assert property (@(posedge scl_out) (bit_cnt == 0) ¦-> (sda_out == tx_data[7])); | FR-PRO-015 | First bit on SDA matches MSB of transmit data | No |
| ASS-06 | Bus Busy Correct Assertion | assert property (@(posedge clk) start_cond ¦-> ##1 bus_busy); | FR-PRO-002 | Bus busy flag asserted one cycle after START | No |
| ASS-07 | Bus Free After STOP | assert property (@(posedge clk) stop_cond ¦-> ##[T_BUF:T_BUF+2] bus_free); | FR-PRO-001, FR-PRO-009 | Bus free asserted after t_BUF following STOP | Expanded |
| ASS-08 | ACK Timing (t_SU_DAT) | assert property (@(posedge scl_out) ack_phase ¦-> $stable(sda_out)[*T_SU_DAT]); | FR-ERR-003 | ACK bit stable for at least t_SU_DAT before SCL rising edge | No |
| ASS-09 | No START on Busy Bus | assert property (@(posedge clk) (start_req && bus_busy && !restart) ¦-> !start_cond); | FR-PRO-006 | No START generated when bus BUSY unless repeated START | Yes |
| ASS-10 | No SDA Drive During ACK Window | assert property (@(posedge clk) tx_ack_phase ¦-> !sda_oe); | FR-ERR-001 | Transmitter releases SDA output enable during ACK window | No |
| ASS-11 | Target ACK on Address Match | assert property (@(posedge scl_out) (addr_ack_phase && addr_match) ¦-> (sda_out == 0)); | FR-ADR-004 | Target pulls SDA LOW on address match in ACK phase | No |
| ASS-12 | Arbitration Loss: No STOP | assert property (@(posedge clk) arb_lost ¦-> !stop_cond); | FR-ERR-012 | Controller must not generate STOP after losing arbitration | Yes |
| ASS-13 | Recovery Pulse Count | assert property (@(posedge clk) recovery_active ¦-> (recovery_cnt <= 9)); | FR-ERR-008 | Recovery pulse count must not exceed 9 | Expanded |
| ASS-14 | SCL Frequency Bound | assert property (@(posedge scl_out) (scl_period >= CLK_DIV_STD)); | FR-SPD-002 | SCL period must be >= Standard-mode divisor (100 kHz max) | No |
| ASS-15 | STOP Only By Controller (NEW v2.0) | assert property (@(posedge clk) stop_cond ¦-> ctrl_active); | FR-PRO-008 | STOP condition can only be generated when controller is active — target cannot generate STOP | No |
| ASS-16 | 1 Bit Per SCL Pulse (NEW v2.0) | assert property (@(posedge scl_out) $rose(scl_out) ¦-> ##1 (bit_cnt == $past(bit_cnt) + 1)); | FR-PRO-012 | bit_cnt increments by exactly 1 on each SCL rising edge — no extra pulses or skipped bits | No |

Formal Verification Targets (v2.0 expanded scope):

- ASS-01: SDA Data Validity — formally proved for all reachable states

- ASS-02: START Condition waveform — formally proved (expanded in v2.0)

- ASS-03: STOP Condition waveform — formally proved (expanded in v2.0)

- ASS-07: Bus Free after t_BUF — formally proved (expanded in v2.0)

- ASS-09: No START on busy bus — formally proved

- ASS-12: No STOP on arbitration loss — formally proved

- ASS-13: Recovery pulse count — formally proved (expanded in v2.0)

# 11. Verification Completion Criteria

Verification is considered complete when ALL of the following criteria are satisfied:

## 11.1 Test Completion

- All 26 directed test cases (TC-01 to TC-26, excluding reserved TC-16 and TC-17) execute and PASS with zero assertion failures.

- All 17 corner case scenarios (CC-01 to CC-17) are exercised and pass.

- Constrained-random tests run for a minimum of 10,000 randomised transactions without errors, using a seed bank of at least 100 seeds.

## 11.2 Coverage Closure

- Functional coverage: 100% of all bins in all 16 covergroups are hit.

- FSM state coverage: 100% — every state in i2c_controller.v and i2c_target.v visited.

- FSM transition coverage: 100% of all legal state transitions exercised.

- Toggle coverage: ≥ 95% on all DUT input and output ports.

- Code coverage (line + branch): ≥ 90% across all RTL modules.

## 11.3 Assertion Status

- All 16 SVA assertions (ASS-01 to ASS-16) pass with zero violations across all tests.

- Formal properties ASS-01, ASS-02, ASS-03, ASS-07, ASS-09, ASS-12, ASS-13 are formally proved with no counterexamples at proof depth ≥ 100 cycles.

## 11.4 Bug Status

- Zero open Severity-1 (blocking) bugs.

- Zero open Severity-2 (major) bugs.

- All Severity-3 (minor) and Severity-4 (cosmetic) bugs reviewed and either fixed or waived with written justification.

## 11.5 Review Sign-off

- Verification Plan review completed and approved by Design Lead and Verification Lead.

- Final simulation regression results reviewed and signed off.

- Formal verification results reviewed and signed off.

- Coverage closure report and Coverage Exclusion File (CEF) generated and approved.

| Criterion | Target | Responsible | Sign-off By |
| --- | --- | --- | --- |
| All directed TCs pass (TC-01 to TC-26) | 100% pass | Verification Eng. | TBD |
| Functional coverage closure (16 covergroups) | 100% bins | Verification Eng. | TBD |
| Toggle coverage | ≥ 95% | Verification Eng. | TBD |
| Code coverage | ≥ 90% | Verification Eng. | TBD |
| Assertion violations | 0 | Verification Eng. | TBD |
| Open S1/S2 bugs | 0 | Design + Verif. | TBD |
| Formal proof complete (7 properties) | No counterexamples | Formal Verif. Eng. | TBD |
| Final regression green | 100% pass | Verification Lead | TBD |
| Coverage Exclusion File approved | All waived items justified | Verification Lead | TBD |

# 12. Assumptions, Constraints and Ambiguity Resolution

## 12.1 Assumptions

- The RTL implementation targets Standard-mode (100 kbit/s) only. Higher speed modes are out of scope.

- A single 50 MHz system clock is used. CLK_DIV_STD = 250 yields correct 100 kHz SCL frequency.

- Pull-up resistors are modelled as weak pull-ups in the testbench. Actual electrical characterisation is performed separately.

- The testbench models SDA/SCL wired-AND topology using tri-state logic. Simulation accurately reflects open-drain bus behaviour.

- The default testbench configuration uses one controller and one or more target devices. Multi-controller arbitration tests are out of scope.

- Spike filtering is modelled as a 3-cycle synchroniser chain in simulation. Board-level EMI testing is outside scope.

- 10-bit addressing features (FR-ADR-005 to FR-ADR-008) are explicitly excluded from all verification activities.

- All timing assertions use parameterised values from i2c_pkg.v. Any parameter change must trigger a full regression re-run.

## 12.2 Constraints

- Simulation is performed using a commercial SystemVerilog simulator (e.g., Synopsys VCS, Cadence Xcelium, or Mentor Questa).

- Formal verification is performed using a commercial formal tool (e.g., JasperGold or VC Formal) with bounded proof depth ≥ 100 clock cycles.

- All RTL must be synthesisable and lint-clean before simulation begins.

- No PLI/VPI black-box models are permitted — all models must be in SystemVerilog or Verilog.

- Coverage database must be saved after each simulation run to allow regression merging.

- Any requirement with status 'Out of Scope' in the FRS shall not generate coverage holes that block sign-off; these must be documented in the CEF.

- A minimum of 100 constrained-random seeds shall be maintained. Passing and failing seeds recorded separately.

## 12.3 Ambiguity Resolution

| Req ID | Original Statement (Ambiguity) | Clarified Interpretation for Verification |
| --- | --- | --- |
| FR-PRO-017 | 'Number of bytes per transfer shall be unrestricted' — not quantitatively testable; RTL has finite loop counter. | Interpreted as: the RTL shall support a minimum of 255 data bytes per transfer without counter overflow. The test suite uses 1, 10, and 255 bytes to bound coverage. |
| FR-PRO-021 | 'Controller is not required to support clock stretch' — contradicts SHOULD in FR-PRO-019/020. | Interpreted as: stretch_en input shall be configurable. When stretch_en=0, clock stretching is disabled. When stretch_en=1, FR-PRO-019/020 apply. Both values must be tested (TC-07 and TC-20). |
| FR-ERR-008 | 'Up to 9 SCL clock pulses' — 'up to' implies it may stop before 9; pass criteria unclear for incomplete recovery. | Interpreted as: the controller shall generate exactly 9 SCL pulses during recovery, monitoring SDA release after each pulse. If SDA is released before pulse 9, recovery terminates early and proceeds to STOP. |
| FR-ERR-014 | Lists three prohibited conditions in one requirement — difficult to achieve individual traceability. | Each condition is formally verified separately. FR-ERR-014 formal property covers all three prohibited concurrent states. Recommended to split into FR-ERR-014a/b/c in future FRS revision. |

# Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-03-10 | Verification Team | Initial release. Generated from FRS-I2C-001 v1.0. All 80 requirements mapped. 15 test cases, 14 coverage items, 14 assertions defined. |
| 2.0 | 2026-03-13 | Verification Team | Updated per review report (2026-03-11). Added TC-18 to TC-26. Added ASS-15 and ASS-16. Added COV-15 and COV-16. Expanded formal scope to 7 properties. Resolved all RTM gaps. Removed multi-controller scenarios from scope. Added UVM architecture diagram and component descriptions. Reformatted all TCs to structured table format. |

END OF DOCUMENT — VPLAN-I2C-001 v2.0